Moving Car Project

Team Members:

Alaa Ibrahim

Bassel Yasser

Sharpel Malek

Sherif Khadr

Contents

INTRODUCTION	1

High Level Design	2

01)	Layered Architecture	2

02)	Modules Description	3

03)	Drivers’ Documentation	4

Low Level Design	14

Figure 1: Project Layered Architecture	2

# INTRODUCTION

In our project we have a four-driving wheel robot and moves in a rectangular shape, on this document we will illustrate the module design and how they integrate with each other, we’ll also discuss the used APIs in more detail and providing the flowchart for each function in each module, and making layered architecture.

In this project we used PWM for controlling motor speed, TIMER on normal mode for controlling the duration of motor, DIO for GPIO Pins and External Interrupt Module for Start / Stop Motor

# High Level Design

## Layered Architecture

Figure 1: Project Layered Architecture

## Modules Description

MCAL Layer:

- DIO:

- External Interrupt: this module takes place in MCAL layer. it controls three external hardware interrupts on pins PD2, PD3, and PB2 which are referred to as INT0, INT1, and INT2 respectively. External interrupts can be level-triggered or edge-triggered.
We program this triggering. INT0 and INT1 can be level-triggered and edge-triggered whereas INT2 can be only edge-triggered.

- Timer: Provides an interface with timer 0 low-level capabilities.

- PWM: a modulation technique that generates variable-width pulses to represent the amplitude of an analog input signal, and directly communicates to hardware.

- PWM Normal: it generates the functionality of PWM using the normal mode of the timer.

HAL Layer:

- Button:

- LED

- HPWM: Is in Middle layer which Application can communicate to MCAL layer.

- HTimer: Provides high-level functions using the lower level timer 0 module capabilities.

- HExtInt: Provides high-level functions using the lower level External interrupt module capabilities.

- HPWM_Normal: : Provides high-level functions using the lower level PWM_NORMAL_ MODE module capabilities.

Service Layer:

Application Layer:

## Drivers’ Documentation

MCAL Layer:

- DIO:

/**

* \brief : This Function Use To Initialize Port Direction Input Or Output

*

* \param ST_pin_config_t _pin_config

*

* \return Std_ReturnType

*/

Std_ReturnType GPIO_pin_direction_intialize(const ST_pin_config_t *_pin_config)

/**

* \brief : This Function USe To Get Direction Of The Pin

*

* \param  ST_pin_config_t  _pin_config

* \param EN_direction_t  direction_status

*

* \return Std_ReturnType

*/

Std_ReturnType GPIO_pin_get_direction_status(const ST_pin_config_t *_pin_config , EN_direction_t *direction_status)

/**

* \brief : This Function Use To Write Logic On The Pin

*

* \param ST_pin_config_t _pin_config

* \param EN_logic_t logic

*

* \return Std_ReturnType

*/

Std_ReturnType GPIO_pin_write_logic(const ST_pin_config_t *_pin_config , EN_logic_t logic)

/**

* \brief : This Function Use To Read Logic On The Pin

*

* \param ST_pin_config_t _pin_config

* \param EN_logic_t logic

*

* \return Std_ReturnType

*/

Std_ReturnType GPIO_pin_read_logic(const ST_pin_config_t *_pin_config , EN_logic_t *logic_status)

- External Interrupt:

// EXT_INT TYPEDEFS

typedef enum EN_EXTINT_ERROR {

EXTINT_OK=0,

EXTINT_NOT_OK

}EN_EXTINT_ERROR;

typedef enum EN_Sence_Control {

LOW_LEVEL=0,

FALLING_EDGE,

RISING_EDGE,

ANY_LOGICAL_CHANGE

}EN_Sence_Control;

typedef enum EN_EXINT_NUMBER{

EXTINT0=0,

EXTINT1,

EXTINT2,

}EN_EXINT_NUMBER;

typedef enum EN_GLOBAL_INT{

DISABLE=0,

ENABLE

}EN_GLOBAL_INT;

// EXT_INT prototypes

/*

Description : This function initializes the GLOBAL_INTERRUPT

ARGS        : takes the state ( ENABLE OR DISABLE )

return      : return EXTINT_OK if the PIN initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR SET_GLOBAL_INTERRUPT(EN_GLOBAL_INT state);

/*

Description : This function initializes the external interrupt number and it's detecting type

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and sense control.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR EXTINT_init(EN_EXINT_NUMBER INTx ,EN_Sence_Control INTxSense);

/*

Description : This function takes the external interrupt number and initialize call back function.

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and pointer to the function we want to execute.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR EXTINT_CallBack(EN_EXINT_NUMBER INTx,void(*ptrfunc)(void));

- Timer:

/**

* \brief Initialize the timer with given mode

* \param u8_a_Mode

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_voidInit(en_TIMMode_t u8_a_Mode);

/**

* \brief Start the timer clock after prescaling it with given value

* \param u8_a_prescaler

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_Start(en_TIM_CLK_SELECT_t u8_a_prescaler);

/**

* \brief Function to stop timer 0

* \return void

*/

void TIM0_Stop();

/**

* \brief Set the timer to start from a certain value

* \param u8_a_FlagValue The value to start the timer from

* \return void

*/

void TIM0_SetValue(Uchar8_t u8_a_startValue);

/**

* \brief Function to get the value of the overflow flag of timer 0

* \param u8_a_FlagValue reference to a variable to store flag value *

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetOVF(Uchar8_t* u8_a_FlagValue);

/**

* \brief Function to clear timer 0 overflow flag

* \return void

*/

void TIM0_ClearOVF(void);

/**

* \brief Function to get the timer state (running/stopped)

* \param u8_a_State reference to a variable to store timer state

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetState(en_TIMState_t* u8_a_State);

/**

* \brief Function to set a function to call when the timer0

*	      Overflow Interrupt is triggered

* \param pv_a_CallbackFn reference to the function to call

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_SetOVFCallback(void (*pv_a_CallbackFn)(void));

- PWM:

  - enu_timer1Status_t Timer1_enuInit (enu_timer1Mode_t)

Author	: Bassel Yasser Mahmoud

Function Name: Timer1_enuInit

Function Description: Initialize Timer1 to Fast PWM Mode

Arguments: copy_enTmerMode {TIMER1_OVF_MODE, TIMER1_FAST_PWM_MODE,}

Return: enu_timer1Status_t {TIMER1_OK or TIMER1_NOK}

  - enu_timer1Status_t Timer1_enuSetPrescallar(enu_timer1Prescalar_t)

Author			: Bassel Yasser Mahmoud

Function Name		: Timer1_enuSetPrescallar

Function Description	: Set Prescaller

Arguments: Timer1_enuSetPrescallar {TIMER1_PRE_1, TIMER1_PRE_64, TIMER1_PRE_256}

Return                 	: enu_timer1Status_t {TIMER1_OK or TIMER1_NOK}

  - enu_timer1Status_t Timer1_enuFastPWMInit(enu_pwm1Mode_t)

Author			: Bassel Yasser Mahmoud

Function Name		: Timer1_enuFastPWMInit

Function Description 	: Set PWM Mode

Arguments: copy_enPWMMode {TIMER1_PWM_NORMAL, TIMER1_PWM_CLR_ON_CMP, TIMER1_PWM_SET_ON_CMP}

Return 			: enu_timer1Status_t {TIMER1_OK or TIMER1_NOK}

  - enu_timer1Status_t Timer1_enuPWMGenerate (Uchar8_t)

Author			: Bassel Yasser Mahmoud

Function Name		: Timer1_enuPWMGenerate

Function Description 	: Generate PWM

Arguments			: copy_u8DutyCycle {1 ~ 100}

Return 			: enu_timer1Status_t {TIMER1_OK or TIMER1_NOK}

  - enu_timer2Status_t Timer2_enuInit (enu_timer2Mode_t)

Author	: Bassel Yasser Mahmoud

Function Name: Timer1_enuInit

Function Description: Initialize Timer2 to Fast PWM Mode

Arguments: copy_enTmerMode {TIMER2_OVF_MODE, TIMER2_FAST_PWM_MODE,}

Return: enu_timer1Status_t {TIMER2_OK or TIMER2_NOK}

  - enu_timer2Status_t Timer2_enuSetPrescallar(enu_timer2Prescalar_t)

Author			: Bassel Yasser Mahmoud

Function Name		: Timer2_enuSetPrescallar

Function Description	: Set Prescaller

Arguments: Timer2_enuSetPrescallar {TIMER2_PRE_1, TIMER2_PRE_64, TIMER2_PRE_256}

Return                 	: enu_timer2Status_t {TIMER2_OK or TIMER2_NOK}

  - enu_timer2Status_t Timer2_enuFastPWMInit(enu_pwm2Mode_t)

Author			: Bassel Yasser Mahmoud

Function Name		: Timer2_enuFastPWMInit

Function Description 	: Set PWM Mode

Arguments: copy_enPWMMode {TIMER2_PWM_NORMAL, TIMER2_PWM_CLR_ON_CMP, TIMER2_PWM_SET_ON_CMP}

Return 			: enu_timer2Status_t {TIMER2_OK or TIMER2_NOK}

  - enu_timer2Status_t Timer2_enuPWMGenerate (Uchar8_t)

Author			: Bassel Yasser Mahmoud

Function Name		: Timer2_enuPWMGenerate

Function Description 	: Generate PWM

Arguments			: copy_u8DutyCycle {1 ~ 100}

Return 			: enu_timer2Status_t {TIMER2_OK or TIMER2_NOK}

- PWM NORMAL:

/*Description : This function selects the normal mode and enables the GLOBAL_INTERRUPT and overflow interrupt for timer2

ARGS        : void

return      : void*/

void timer2_init(void);

/*Description : This function selects the prescaler (clk/1024). the timer start counting once we call this function.

ARGS        : void

return      : void*/

void timer2_start(void);

/*Description : This function selects the no clock source option. the timer stop counting once we call this function.

ARGS        : void

return      : void*/

void timer2_stop(void);

/*Description : This function calculate the on time based on duty cycle we need .

ARGS        : takes the duty cycle

return      : void*/

void timer2_set_pwm_normal(Uchar8_t dutycycle);

HAL Layer:

Button:

/**

* \brief : This Function Use To Initialize Button Pin And Port

*

* \param ST_PUSH_BTN_t  *btn

*

* \return Std_ReturnType

*/

Std_ReturnType PUSH_BTN_intialize(const ST_PUSH_BTN_t *btn)

/**

* \brief : This Function Will Use To Read Button Statues If It Is Pressed Or Releassed

*

* \param ST_PUSH_BTN_t  *btn

* \param EN_PUSH_BTN_state_t  *btn_state

*

* \return Std_ReturnType

*/

Std_ReturnType PUSH_BTN_read_state(const ST_PUSH_BTN_t *btn , EN_PUSH_BTN_state_t *btn_state)

- LED:

/**

* \brief : This Function Is Used To Initialize Pin And Port Led

*

* \param ST_LED_t *led

*

* \return Std_ReturnType

*/

Std_ReturnType LED_initialize(const ST_led_t *led)

/**

* \brief : This Function Used To Turn Led On

*

* \param ST_led_t *led

*

* \return Std_ReturnType

*/

Std_ReturnType LED_turn_on(const ST_led_t *led)

/**

* \brief : This Function Used To Turn Led Off

*

* \param ST_led_t *led

*

* \return Std_ReturnType

*/

Std_ReturnType LED_turn_off(const ST_led_t *led)

/**

* \brief : This Function Used To Toggle Led

*

* \param ST_led_t *led

*

* \return Std_ReturnType

*/

Std_ReturnType LED_toggle(const ST_led_t *led)

- Motor

/**

* \brief initialize motor pins

* \param pst_a_Motor reference to desired motor

* \return en_MotorError_t

*/

en_MotorError_t DCM_Init(st_Motor_t *pst_a_Motor);

/**

* \brief Function to start the given motor

* \param pst_a_Motor reference to desired motor

* \return en_MotorError_t

*/

en_MotorError_t DCM_Start(st_Motor_t *pst_a_Motor);

/**

* \brief Function to stop the given motor

* \param pst_a_Motor reference to desired motor

* \return en_MotorError_t

*/

en_MotorError_t DCM_Stop(st_Motor_t *pst_a_Motor);

- HPWM:

  - enu_pwmStatus_t pwm_enInit(void)

Author			: Bassel Yasser Mahmoud

Function Name		: pwm_enInit

Function Description : Initialize PWM to be fast PWM, set prescaller, Set PWM Mode

Arguments			: void

Return 			: enu_pwmStatus_t {PWM_OK or PWM_NOK}

  - enu_pwmStatus_t pwm_enGenerate(Uchar8_t)

Author			: Bassel Yasser Mahmoud

Function Name		: pwm_enGenerate

Function Description 	: Generate PWM signal

Arguments			: Uchar8_t

Return 			: enu_pwmStatus_t {PWM_OK or PWM_NOK}

- HTimer:

/**

* \brief Generate Synchronous delay (busy waiting)*

* \param Copy_delayTime Desired delay

* \param Copy_timeUnit Time units (Seconds, mSeconds, uSeconds)

*

* \return en_HTIMErrorState_t

*/

en_HTIMErrorState_t TIM0_SyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit);

/**

* \brief Generates delay asynchronously

* \param u32_a_delay desired delay

* \param u8_a_timeUnit delay time units

* \param Copy_pvCallbackFn function to call when delay is complete

*

* \return en_TIMErrorState_t

*/

en_HTIMErrorState_t TIM0_AsyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit, void (*Copy_pvCallbackFn)(void));

/**

* \brief Function to end a delay asynchronously

* To Stop Async Delay: No Restrictions

* To Stop Sync Delay: should only be called in an ISR/Callback function

*

* \return void

*/

void TIM0_AsyncEndDelay();

- HExtInt:

/*

Description : This function initializes the external interrupt number and it's detecting type and initialize call back function.

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and sense control and and pointer to the function we want to execute when interrupt occurs.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR H_EXTINT_create(EN_EXINT_NUMBER INTx ,EN_Sence_Control INTxSense,void(*ptrfunc)(void));

- HPWM_NORMAL:

/*

Description : This function selects the normal mode and enables the GLOBAL_INTERRUPT and overflow interrupt for timer2

ARGS        : void

return      : void

*/

void H_PWM_NORMAL_init(void);

/*

Description : This function selects the no clock source option. the timer stop counting once we call this function.

ARGS        : void

return      : void

*/

void H_PWM_NORMAL_stop(void);

/*

Description : This function calculate the on time based on duty cycle we need . then start the timer

ARGS        : takes the duty cycle

return      : void

*/

void H_PWM_NORMAL_setDutyCycle(Uchar8_t dutycycle);

Application Layer:

/**

* \brief Function to move car forward

* \return void

*/

static void APP_moveForward(void);

/**

* \brief Function to stop the car

* \return void

*/

static void APP_Stop(void);

/**

* \brief Function to rotate car right

* \return void

*/

static void APP_rotate(void);

/**

* \brief Function to call in EXTI1 ISR when button 0 is pressed

* \return void

*/

void btn0_callback(void);

/**

* \brief Function to call in EXTI0 ISR when button 1 is pressed

* \return void

*/

void btn1_callback(void);

/**

* \brief Initialize all used modules

* \return void

*/

void initMain(void);

/**

* \brief  Application main logic

* \return void

*/

void appMain(void);

# Low Level Design

MCAL Layer:

- DIO:

- External Interrupt:

- Timer:

TIM0_Init

TIM0_Start

TIM0_Stop

- PWM:

  - enu_timer1Status_t Timer1_enuInit (enu_timer1Mode_t)

  - enu_timer1Status_t Timer1_enuSetPrescallar(enu_timer1Prescalar_t)

  - enu_timer1Status_t Timer1_enuFastPWMInit(enu_pwm1Mode_t)

  - enu_timer1Status_t Timer1_enuPWMGenerate (Uchar8_t)

  - enu_timer2Status_t Timer2_enuSetPrescallar(enu_timer2Prescalar_t)

  - enu_timer2Status_t Timer2_enuInit (enu_timer2Mode_t)

  - enu_timer2Status_t Timer2_enuFastPWMInit(enu_pwm2Mode_t)

  - enu_timer2Status_t Timer2_enuPWMGenerate (Uchar8_t)

- PWM NORMAL:

HAL Layer:

- Button:

- LED:

- Motor

- HPWM:

  - enu_pwmStatus_t pwm_enInit(void)

  - enu_pwmStatus_t pwm_enGenerate(Uchar8_t)

- Timer0

HTIM_SyncDelay

Application Layer:

- APP_Init

APP_Stop

APP_moveForward

- Main App State Machine